PAGE  
209

Identify Heating and A/C Concerns

Meets ASE Task:  (A7-A-1) P-1, (A7-C-3) P-2, (A7-D-7) P-2  Identify and interpret heating and A/C concerns; determine needed action.

Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1                   
_____  1.  Check the front of the radiator and air-conditioning condenser for debris that could 


     limit airflow.  Clean as required.



OK_____

NOT OK_____

_____  2.  Perform a thorough visual inspection of the cooling system.  Look for hoses that may 


     be leaking, cut, or swollen.



OK_____

NOT OK_____

_____  3.  Start the engine and operate the heater and air-conditioning controls for proper 


     operation including:



defroster (airflow to windshield)
OK_____

NOT OK_____



heater (airflow to floor) 

OK_____

NOT OK_____



A/C (airflow to vents) 

OK_____

NOT OK_____



blower motor on all speeds

OK_____

NOT OK_____

_____  4.  Using an infrared pyrometer, measure the temperature of the upper radiator hose.



Temperature = ________ (should be close to the same temperature as the 



thermostat rating)



OK_____

NOT OK_____

_____  5.  Based on the tests, what is the needed action:  ________________________________

