Oxygen Sensor Diagnosis

Meets ASE Task:  (A8-B-7) P-2  Inspect and test sensors, actuators, and circuits using a graphing multimeter (GMM)/digital storage oscilloscope (DSO); perform necessary action.

Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1

_____  1.  Connect the scan tool to the DLC and start the engine.

_____  2.  Operate the engine at a fast idle (2500 RPM) for

2 minutes to allow time for the oxygen sensor to

warm to operating temperature.

_____  3.  Observe the oxygen sensor activity on the scan tool

to verify closed loop operation.

_____  4.  Select “snap shot” mode and hold the engine speed steady and start recording.

_____  5.  Play back snap shot and place a mark beside each range of oxygen sensor voltage for

each frame of the snap shot.

Between 0 and 300 mV    Between 300 and 600 mV   Between 600 and 1000 mV

____________________   ______________________   _______________________

(record # of times)               (record # of times)                 (record # of times)

_____  6.  Results:  A good oxygen sensor and computer system should result in most snap shot

values at both ends (0 to 300 and 600 to 1000 mV).  If most of the readings are in the

middle, the oxygen sensor is not working correctly.

OK ________    NOT OK ________

_____  7.  Based on the test results, what is the necessary action? _________________________

_________________________________________________________________