Fuel Trim Diagnosis

Meets ASE Task:  (A8-B-6) P-1  Diagnose emissions or driveability concerns without stored diagnostic trouble codes; determine necessary action.

Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1

Fuel trim is the computer correction factor that uses the oxygen sensor to determine if more or less fuel needs to be delivered by the fuel injectors.  Fuel trim is only available on a scan tool.

_____  1.  Connect a scan tool and select long-term

fuel trim (LTFT) (block learn) and short-

term fuel trim (STFT).

_____  2.  Start the engine and operate until normal

operating temperature and closed loop

status is achieved.

_____  3.  Record the following cell number and

LTFT amount:

Cell                LTFT          STFT

Idle in Drive (if automatic transmission only)	________	________   ________

Idle in Park A/C off	________	________   ________

Idle in Park A/C on	________	________   ________

3000 RPM in Park     	________	________   ________

Results:  Fuel trim should be within plus or minus 10% or within 118-138 if the

block/integration is displayed as a binary number.

_____  4.  Based on the test results, what is the necessary action?  ________________________

________________________________________________________________