Smoke Test of the EVAP System

Meets ASE Task:  (A8-E-5) P-2  Inspect and test components and hoses of the evaporative emissions control system and DTCs; perform necessary action.

Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1

_____  1.  Check for any EVAP-related diagnostic trouble codes (DTCs).

_________________________________________________________________

_____  2.  Check service information for the procedures and pressures to follow when checking

the evaporative emission control system for leaks using smoke.

__________________________________________________________________

__________________________________________________________________

Specified maximum pressure = _________________________________

_____  3.  Connect the smoke machine to the evaporative emission control system following the

instructions supplied with the smoke machine.

____________________________________

____________________________________

____________________________________

____________________________________

_____  4.  Use a bright light and look for smoke leaking from the evaporative emission control

system.  Describe the leaks, if any.

__________________________________________________________________

__________________________________________________________________

_____  5.  Based on the results of the scan tool and smoke testing, what is the necessary action?

__________________________________________________________________