Starter Disassembly and Testing

Meets ASE Task:  Not specified by ASE

Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1

_____  1.  Clean and visually inspect the starter for physical damage such as a cracked or broken

drive-end housing.

OK _____ 	NOT OK _____

_____  2.  Mark the location of the through bolts on the frame housing and make note of the

location of a special through bolt used to retain a support bracket.

_____ used a support bracket		_____ did not use a support bracket

_____  3.  Remove the solenoid (if it is a solenoid activated-type starter).

CAUTION:  When removing the solenoid, use caution because the plunger return spring may cause the solenoid to be forced away from the starter when the attaching bolts (nuts) are removed.

_____  4.  Remove the through bolts.

_____  5.  Gently remove the brush-end housing and armature from the field housing.

_____  6.  Remove the shift fork and separate the drive-end housing from the field housing.

_____  7.  Visually inspect all the parts and note their condition.

armature			OK ____   NOT OK ____ Notes: ___________________

brushes/holders		OK ____   NOT OK ____ Notes: ___________________

field coils			OK ____   NOT OK ____ Notes: ___________________

drive-end housing		OK ____   NOT OK ____ Notes: ___________________

shift lever			OK ____   NOT OK ____ Notes: ___________________

_____  8.  Set the multimeter to read ohms and check the resistance between the hot brushes and

the field housing (should read OL).

OK _____	NOT OK _____

_____  9.  Check the armature on a growler for opens and shorts.

OK _____	NOT OK _____