Starter Relays and Solenoids

Meets ASE Task:  (A6-C-3) P-2 and (A6-C-5) P-2  Inspect and test starter relays, solenoids, connections, and wires; determine necessary action.

Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1

_____  1.  Clean and visually inspect the starter solenoid and/or relay for physical damage.

OK _____ 	NOT OK _____

_____  2.  Set a digital multimeter (DMM) to read ohms (low scale) and check the hold-in coil

and the pull-in coil.

Pull-in coil.  Measure between terminals “S” and “M”:

resistance = ______ (should be 0.2 to 0.4 ohm)  OK _____	NOT OK _____

Hold-in coil.  Measure between terminals “S” and the solenoid housing:

resistance = _______ (should be 0.4 to 0.6 ohm) OK _____	NOT OK _____

_____  3.  Test the pull-in winding by applying 12 volts to terminal “S” and ground to terminal

“M.”  Check that the plunger will be drawn into the solenoid.

OK _____	NOT OK _____

_____  4.  Check the hold-in winding by connecting 12 volts

to terminal “S” and the other wire to ground.  The

plunger should be drawn into the solenoid housing.

OK _____	NOT OK _____

_____  5.  Measure coil resistance of the relay (terminals 86 and

85).

Resistance = ______ ohms

(should be 60 to 100 ohms)

OK _____	NOT OK _____

_____ 6.  What is the necessary action?

_____________________________________

_____________________________________