MAF Sensor Diagnosis

Meets ASE Task:  (A8-B-7) P-2  Inspect and test sensors, actuators, and circuits using a graphing multimeter (GMM)/digital storage oscilloscope (DSO); perform necessary action.

Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1

A "Mass Air Flow" sensor produces a variable output depending on the MASS of the air flow through the sensor.  A faulty MAF can cause driveability problems and stalling.  A good MAF sensor should produce a signal that increases with engine speed.

_____  1.  Check service information for the specified procedure to follow to test the MAF

sensor.

__________________________________________________________________

_____  2.  Use a meter or scope with a frequency counter to record

frequency or voltage at idle and at WOT (short bursts).

at idle = ____________       at WOT = _____________

_____  3.  Use a scan tool and record grams per second.

at idle = ____________       at WOT = _____________

A good MAF should read:

- greater than 100 grams per second (scan tool diagnosis)

- higher than 7000 Hertz (7 KHz) (digital MAF)

- higher than 4 volts (analog MAF)

_____  4.  If the MAF sensor reading does not exceed these values, the sensing wire may be

contaminated or the sensor itself is defective.

_____  5.  Based on the test results, what is the necessary action?  ________________________

_________________________________________________________________

_________________________________________________________________