PAGE  
129
[image: image2.jpg]' ‘ ) _ 2ms<SDIV
SAVE RECALL
SRVYE CLEAR
= RAPLY OPREEN S REMDI | RINT








Fuel Pump Current Ramping
Meets NATEF Task:  (A8-D-3)  Inspect and test fuel pumps and pump control systems for pressure, regulation and volume; perform necessary action.  (P-1)
[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
[image: image2.jpg]        
_____  1.  Check service information for the specified fuel pump operating speed (normally from 

     3500 to 7500 RPM).


______________________________________

_____  2.  Tools and equipment needed to determine 

  
     the rotational speed of the fuel pump include:


____  Digital storage oscilloscope


____  Current clamp


____  Fused jumper lead


_____  3.  Perform a current ramping procedure and determine the following:



____  Number of commentator segments = ______ (usually 8, 10, or 12)



____  Milliseconds needed for one rotation of the pump = ______



____  Fuel pump speed = 60,000 ÷ milliseconds = _______


_____  4.  Based on the test performed, what is the necessary action?  ___________________



_______________________________________________________________


