IAC Scan Tool Diagnosis

Meets NATEF Task:  (A8-B-8)  Perform active tests of actuators using a scan tool; determine necessary action.  (P-3)

Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1

_____  1.  Check service information for the specified IAC position as displayed on a scan tool.

__________________________________________________________________

__________________________________________________________________

_____  2.  Check service information for the units used to express the position of the IAC.

_____ Counts

_____  Percentage (%)

_____  Milliamperes (mA)

_____  Other (describe) ______________________________________________

_____  3.  What is the IAC position on a fully warmed engine without any accessories on?

__________________________________________________________________

_____  4.  Create a vacuum leak.  How does the IAC react?

____  IAC position decreased (usually for speed density-equipped engines)

____  IAC position increases (usually for engines equipped with a MAF sensor)

_____  5.  Based on the test results, what is the necessary action?  ________________________

_________________________________________________________________