PCV System Test

Meets NATEF Task:  (A8-E-2)  Inspect, test and service positive crankcase ventilation (PCV) filter/breather cap, valve, tubes, orifices, and hoses; perform necessary action.  (P-2)

Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1

The purpose of the "positive crankcase ventilation" (PCV) system is to use engine intake manifold vacuum to draw crankcase vapors that occur due to blow-by past the piston rings into the engine to be burned.

NOTE:  If there is gasoline in the engine oil, gasoline vapors will be drawn into the engine and

the intake charge will be richer.  Therefore, if black exhaust smoke or a rich condition is

being diagnosed, check the crankcase for the presence of gasoline.

A good PCV system should draw fresh air into the engine through a filter usually located in the air cleaner or on the valve (cam) cover.  This filtered air then is mixed with the blow-by gases in the crankcase and through a PCV valve which regulates the flow into the engine.

PCV Valve Operation:

a.   Idle = high vacuum = PCV valve almost closed by high vacuum.

b.   At cruise = 10-15 in. Hg. = PCV valve is opened to allow the crankcase gases to be

drawn into the engine (vacuum and internal spring are almost balanced).

c.   At wide open throttle (WOT) low vacuum =  PCV valve is fully opened by internal

spring permitting maximum flow.

Testing the System:

_____  1.  Start the engine and allow it to idle.

_____  2.  Remove the oil fill cap.

_____  3.  Place a piece of paper or a 3" x 5" card over the filler.  (The PCV system is

functioning correctly if the paper is held down tight onto the filler by vacuum in the

crankcase).

_____  4.  Seal off the oil fill opening and measure the crankcase vacuum at the dipstick tube

= ________ (should be about 0.5 in. Hg. or 7 in. or more of water if using a water

manometer).

OK _____    NOT OK _____