Machining a Brake Drum

Meets NATEF Task:  (A5-C-3) Refinish brake drum; measure final drum diameter. (P-1)

Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1

_____  1.  Measure the drum and double check that the brake drum can be safely machined.

- Maximum allowable inside diameter = ________

- Actual measurement of the drum = _____  left = _______  right = _______

OK to machine _______	discard ________

_____  2.  Select the proper tapered centering cone and face supporting plate.

_____  3.  Install a self-aligning spacer (SAS) and tighten the spindle nut.

_____  4.  Perform a scratch cut.

_____  5.  Stop the lathe, loosen the spindle nut, and rotate the brake

drum 180 (one-half turn) and retighten the spindle nut.

_____  6.  Perform a second scratch cut.

- If the second cut is in the same location, proceed with machining.

- If the second cut is on the opposite side of the drum, clean or repair the lathe before machining.

_____  7.  Install a silencer band (vibration damper).

_____  8.  Machine the drum.

_____  9.  The measurement of the drum after machining = _________.

Does this allow 0.030” or more for wear?

Yes ____  (install on the vehicle)       No ____ (replace the drum)