Diagnose ABS System Concerns

Meets NATEF Task:  (A5-G-4) Diagnose ABS operational concerns; determine necessary action. (P-2)

Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1

_____  1.  Poor stopping – Check the following system or components.

A.  Tires – condition and sizes	OK _____     NOT OK _____

B.  Base brake components such as calipers, pads, and drive brake components

OK _____     NOT OK _____

_____  2.  Abnormal pedal feel or pulsation – Check the following components.

A.  Wheel speed sensor tone ring or wiring for damage.

OK _____     NOT OK _____

B.  Brake rotors and drum for out-of-round or other faults

OK _____     NOT OK _____

C.  Master cylinder and brake fluid for level or contaminants

OK _____     NOT OK _____

_____  3.  Wheel lockup – Check the following components.

A.  Base brake friction material (pads and linings) for grease, excessive wear or

contamination.

OK _____     NOT OK _____

B.  Wheel speed sensor tone ring for damage

OK _____     NOT OK _____

C.  Excessively worn or mismatched tires

OK _____     NOT OK _____

_____  4.  Abnormal noise – Check the following components.

A.  Accumulator leakage creating the need for extended pump operation

OK _____     NOT OK _____

B.  Base brakes for excessive wear or defective friction components

OK _____     NOT OK _____

_____  5.  Based on the diagnosis, what is the necessary action? _________________________