PAGE  
110
[image: image1.jpg]We Suppont

EEEEEEEEE








Tie-Rod End Inspection and Replacement

Meets NATEF Task:  (A4-B-17) Inspect, replace, and adjust tie rod ends (sockets), tie rod sleeves and clamps.  (P-1)
[image: image1.jpg]Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
_____  1.  Verify that the tie-rod end(s) requires replacement.  Check all that apply.



_____ Torn grease boot



_____ Joint has side-to-side movement



_____ Physically damaged



_____ Other (specify) _______________________________________________

_____  2.  Hoist the vehicle safely.

_____  3.  Compare the replacement tie-rod end with the original to be sure that the new end is 

     correct.

_____  4.  Remove the retaining nut and use a tie-rod puller to separate the tie-rod end from the 

     steering knuckle and/or center link.

HINT:  Often a hammer can be used to jar loose the tie-rod end especially if a downward force is exerted on the tie-rod while an assistant taps on the steering knuckle at the tie-rod end.

_____  5.  Measure the distance between the center of the tie-rod end and the adjusting sleeve 

     and record this distance so the replacement tie-rod end can be installed in 

     approximately the same location so that the wheel alignment (toe setting) will be close 

     to being correct.

_____  6.  Unscrew the old tie-rod end and discard.

_____  7.  Install the replacement tie-rod end and adjust to the same distance as measured and 

     recorded in #5.

_____  8.  Install the tie-rod end onto the steering knuckle and torque the retaining nut to factory 

     specifications.



Torque specifications for the tie-rod retaining nut = ___________

_____  9.  Lower the vehicle and align the vehicle before returning it to the customer.


