PAGE  
14
[image: image2.jpg]STOP LIGHT
SWITCH

PEDAL HEIGHT








Brake Pedal Height

Meet NATEF Task:  (A5-B-2) Measure brake pedal height; determine necessary action. (P-1)

[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
_____  1.  State the vehicle manufacturer’s specified brake height testing procedure:



__________________________________________________________________



__________________________________________________________________



__________________________________________________________________

[image: image2.jpg]​​​​_____  2.  Measure the brake pedal height from the bottom

                 of the steering wheel or floor to the brake pedal.

________ = inch (cm)

_____  3.  Depress the brake pedal until the brakes are 

                 applied and measure the brake pedal height.

________ = inch (cm)

_____  4.  Subtract the second reading from the first 

                 reading.  This is the brake pedal travel.

_______ = brake pedal travel (should be a maximum of 2.0 to 2.5 in.)

_____  5.  List three items that could cause a greater than normal brake pedal travel.



A.  _______________________________



B.   _______________________________



C.   _______________________________

_____  6.  Based on the test results, what is the necessary action: _________________________



_________________________________________________________________

