PAGE  
27
[image: image2.jpg]" ¢
cL o
HI -
veo, i
L mo
0T0! A
":IE?:GULE
Poun

nn









Brake Fluid  
Meet NATEF Task:  (A5-B-9) Select, handle, store, and fill brake fluids to proper level.  (P-1)

[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
_____  1.  Consult the vehicle manufacturer’s service information and determine the specified

     type of brake fluid.

____ DOT 3
   ____ DOT 4        ____ Other (specify) _____________________

_____  2.  All brake fluid should be stored in a sealed container.  Specify what type and size 

[image: image2.jpg]
     container of container is being used.

_____ Metal (preferred because air containing moisture cannot

           penetrate metal)

_____ Plastic (makes shelf life shorter because air containing

           moisture can penetrate most plastic)

_____ Size (number of ounces or ml) _____________

_____  3.  Brake fluid can remove paint so protective covers should be used whenever handling 


     brake fluid.  Check all that should be done when handling brake fluid.

_____ Use fender covers

_____ Wear protective gloves

_____  4.  Fill brake fluid to the “MAX” line on the master cylinder reservoir.

CAUTION:  If the brake fluid level is too high, the brakes may self-apply when 

the normal operation of the wheel brakes warms the brake fluid, which expands in 

volume.  If the brake fluid is unable to expand in the master cylinder reservoir, the 

pressure increases and the brakes can be applied even though the driver did not 

depress the brake pedal.
