PAGE  
29
[image: image2.jpg]







Manual Brake Bleeding

Meets NATEF Task:  (A5-B-12) Bleed and/or flush brake system.  (P-1)

[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
_____  1.  Check the service information for the specified brake bleeding procedure for the 

                 vehicle being serviced.

_____  2.   Fill the master cylinder reservoir with clean brake fluid from a sealed container.

[image: image2.jpg]
_____  3.  Hoist the vehicle safely.

_____  4.  Open the right rear bleeder valve and 


     have an assistant slowly depress the 


     brake pedal to bleed the wheel cylinder/caliper.


     Close the bleeder valve and have the assistant 


     slowly release force on the brake pedal.  Wait 15 


     seconds and repeat the process until a solid stream 


     of brake fluid is observed.

_____  5.  Repeat the bleeding procedures for the left-rear, 


     right-front, and then the left-front wheel brakes.

_____  6.  After all four wheel brakes have been bled, lower the vehicle and fill the master 


     cylinder to the full mark.

CAUTION: Check the master cylinder reservoir frequently and refill as necessary with clean brake fluid.  Do not overfill a master cylinder reservoir.

_____  7.  Test drive the vehicle checking for proper brake operation before returning the vehicle 


     to the customer.

