PAGE  
33
[image: image1.jpg]We Suppont

EEEEEEEEE








Surge Brake Bleeding  

Meets NATEF Task:  (A5-B-12) Bleed and/or flush the brake system.  (P-1)
[image: image1.jpg]Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
_____  1.   Check the service information for the specified brake bleeding procedure for the 


      vehicle being serviced.



________________________________________________________________



________________________________________________________________



________________________________________________________________
_____  2.  Slip the plastic hose over the bleeder screw of the wheel cylinder or caliper to be 

     bled and submerge the end of the tube in the jar of brake fluid.

_____  3.  Open the bleeder screw approximately one-half turn.

_____  4.  With the bleeder screw open, have your assistant rapidly pump the brake pedal several 


     times.  Air bubbles should come out with the brake fluid.

_____  5.  While your assistant holds the brake pedal to the floor, close the bleeder screw.

_____  6.  Repeat steps 2 through 4 at each bleeder screw in the recommended order.

_____  7.  Re-bleed the system using one of the four other methods described above.

