PAGE  
36
[image: image2.jpg]







Wheel Bearing Service 

Meets NATEF Task:  (A5-F-2) Remove, clean, inspect, repack, and install wheel bearings and replace seals; install hub and adjust bearings.  (P-1)

[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
_____  1.  Remove the wheel cover and the hub dust cap (grease cap).

[image: image2.jpg]
_____  2.  Remove and discard the cotter key.

_____  3.  Remove the spindle nut, washer and outer 


     bearing.

_____  4.  Remove inner and outer bearing and grease 

 
     seal.

_____  5.  Thoroughly clean the bearing in solvent and 


     denatured alcohol or brake cleaner and blow 


     it dry with compressed air.

_____  6.  Closely inspect the bearing for wear or damage.

_____  7.  Show the instructor the cleaned bearing.      
Instructor's OK _______________

_____  8.  Repack the bearing with the correct type of wheel bearing grease.

_____  9.  Install a new grease seal using a seal installing tool.

_____ 10. Correctly adjust the bearing preload:

____  Install the spindle nut and while rotating the tire assembly, tighten (snug 

          only, 12 to 30 lb.-ft.) with a wrench to "seat" the bearing correctly in the 

          race.

_____ While still rotating the tire assembly, loosen the nut approximately 1/2 turn 

           and then hand tighten only.

_____ Install a new cotter key (the common size is 1/8” diameter and 1.5 inches 

           long).

_____ Bend the ends of the cotter key up and around the nut to prevent 

           interference with the dust cap.

_____ 11.  Install the hub dust cap (grease cap) and wheel cover.

