PAGE  
39
[image: image2.jpg]







Sealed Wheel Bearing Replacement 

Meets NATEF Task:  (A5-F-7) Remove and reinstall sealed wheel bearing assembly. (P-2)

[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
_____  1.  Check service information for the specified replacement procedure for the vehicle 


     being serviced. _______________________________________________________



________________________________________________________________

_____  2.  Loosen (do not remove) the drive axle shaft nut.

_____  3.  Hoist the vehicle safely to a good working height (about chest high).

_____  4.  Remove the front wheel.

_____  5.  Use a steel drift between the caliper and the rotor cooling vent hole to hold the rotor 

                 from rotating.

_____  6.  Remove the drive axle shaft hub nut.

_____  7.  Remove the front disc brake caliper.

_____  8.  Remove the rotor, the hub, and splash shield retaining bolts.

_____  9. Mark the location of the hub and make certain the hub is loose on the steering knuckle.

[image: image2.jpg]
_____ 10. Install the hub puller and remove the


     bearing and hub assembly.

_____ 11. Clean and lubricate hub bearing surface.

_____ 12. Reinstall the hub and bearing using the 


     drive axle shaft nut.  (Do not torque to the 


     final setting, just until the hub is seated.)

_____ 13. Reinstall the rotor, caliper, and wheel.

_____ 14. Lower the vehicle and tighten the drive axle shaft nut to the final specification.

Specification = _______________ (usually about 200 lb.-ft.)

