PAGE  
42
[image: image2.jpg]et UL EEEETEY

I,“ﬁ.m.y. )
r









Drum Brake Inspection 

Meets NATEF Task:  (A5-C-4) Remove, clean, inspect drum brake parts; determine necessary action. (P-1)

[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
_____  1.  Hoist the vehicle safely to a good working height (about chest high).

_____  2.  Remove the rear wheels.  

_____  3.  Remove the brake drums (they should pull straight off - if you have problems, see 

                 the instructor).

_____  4.  Check the thickness of the lining remaining.  (The brake lining should show

                 equal thickness on both shoes and have a minimum thickness equal to the 

                 thickness of a nickel.)



OK ____     NOT OK ____     Describe any faults _____________________

_____  5.  Tap the brake drum with a steel hammer (it should ring like a bell)



OK ____     NOT OK ____ (discard)

_____  6.  Measure the inside diameter of the drum and compare to the specifications.

[image: image2.jpg]


Specifications = _________________________



Actual:  left = _____     right = _____



OK ____     NOT OK ____

_____  7.  Check for any brake fluid or rear axle fluid leakage.



OK ____     NOT OK ____

_____  8. Based on this inspection, what is the necessary action?



_________________________________________________________________



_________________________________________________________________
