PAGE  
48
[image: image2.jpg]







Install Wheel and Torque Lug Nuts

Meets NATEF Task:  (A5-A-4) Install wheel and torque lug nuts and make final checks and adjustments. (P-1)

[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
_____  1.  Check service information and determine the vehicle manufacturer’s specified lug nut 

     torque specification.



___________ (usually between 80 and 100 lb-ft)

_____  2.  Use a hand-operated wire brush on the wheel studs to ensure clean and dry threads and 


     check for damage.



OK _____     NOT OK _____  Describe fault: ____________________________

_____  3.  Verify that the lug nuts are OK and free of defects.

_____  4.  Install the wheel over the studs and start all lug nuts (or bolts) by hand.

_____  5.  Tighten the lug nuts a little at a time in a star pattern using an air impact wrench 


     equipped with the proper torque limiting adapter or a torque wrench.

[image: image2.jpg]

_____ Used a torque wrench



_____ Used an air impact with a torque 

           limiting adapter (torque stick)

_____  6.  Tighten the lug nuts to final torque in a star pattern.

NOTE:  “Tighten one, skip one, tighten one” is the usual method if four or five 

lug nuts are used. 

