PAGE  
54
[image: image2.jpg]







Disc Brake Caliper Overhaul
Meets NATEF Task:  (A5-D-5) Disassemble and clean caliper assembly, inspect parts for wear, rust, scoring, and damage; replace seal, boot, and damaged or worn parts.   (P-1)

[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
_____  1.  Check the service information for the specified disc brake caliper overhaul procedure.


______________________________________________
[image: image2.jpg]
_____  2.  Hoist the vehicle safely and remove the wheels.

​​​​​​_____  3.  Open the bleeder valve and compress the piston.

NOTE:  If the bleeder valve breaks or if the 

piston does not retract, consider replacing the

caliper instead of overhauling it.

_____  4.  Remove the brake line and the caliper assembly from

     the vehicle.

NOTE:  Remove one caliper at a time to avoid the possible problem of installing the caliper on the wrong side of the vehicle.

_____  5.  Place a block of wood or a shop cloth beside the caliper piston and use compressed air 


     to remove the piston, dust boot, and caliper O-ring.

_____  6.  Clean the caliper assembly and piston.     
_____Piston OK 
_____Piston pitted 

_____  7.  Thoroughly coat the new square-cut O-ring and install it in the groove in the caliper 


     housing.

_____  8.  Install the piston into the caliper.

_____  9.  Install the caliper using new disc brake pads as necessary and new copper washers on 


     both sides of the banjo bolt, if equipped.

_____ 10. Bleed the caliper and repeat on the other side of the vehicle.

_____ 11. Depress the brake pedal to ensure a firm brake pedal and test for proper brake 

                 operation.
