PAGE  
58
[image: image2.jpg]o)
W DRILL —)
BIT OR DOWEL ‘









Rear Disc Parking Brake Adjustment

Meets NATEF Task:  (A5-D-10) Retract caliper piston on an integrated parking brake system. (P-3)

[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
Many vehicles equipped with rear disc brakes use a mechanical activated parking brake that is integral with the caliper.  Most are designed to be self-adjusting by adjusting when excessive brake pad-to-rotor clearance occurs.

_____  1.  Check the service information for the specified rear disc brake parking brake 
     adjustment procedure. 


_________________________________________________________________
_____  2.  Check the number of “clicks” of the parking brake.



_____ Number of clicks (should be between 3 and 9)



OK _____     NOT OK _____


     If over 10 clicks is needed to set the parking brake, the rear disc brake caliper needs 

[image: image2.jpg]
     adjustment.

_____  3.  Hoist the vehicle safely and remove both rear wheels.

_____  4.  Carefully inspect the rear disc brakes for damage and 

     measure the pads for excessive wear.









OK _____     NOT OK _____  

Replace the pads if worn to the minimum allowable thickness.

_____  5.  If the disc brake pads are serviceable, operate the parking brake lever using the 

 
     appropriate size wrench on the actuating arm retaining bolt/nut while lightly tapping 


     on the caliper using a dead blow plastic hammer.  The adjusting mechanism should 


     cause the piston to be repositioned with the correct pad to rotor clearance.



OK _____     NOT OK _____


     If the proper clearance is not achieved, replacement of the calipers is required.

