PAGE  
62
[image: image1.jpg]We Suppont

EEEEEEEEE








Brake Drum Measurement

Meets NATEF Task:  (A5-C-2) Remove, clean, inspect, and measure brake drums; determine necessary action. (P-1)

[image: image1.jpg]Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
_____  1.  Wet the brake drum or use an enclosure to help 


     protect against asbestos exposure.

_____  2.  Remove the brake drum from the vehicle and 

     label the left and right to ensure that the

     drum is replaced in the original location.

_____  2.  Thoroughly inspect the brake drum.

· Hot (hard) spots


OK_____    NOT OK_____ (requires replacement)

· Tap with a hammer.  The brake drum should ring like a bell if not cracked.


OK_____    NOT OK_____  (requires replacement)

_____  3.  Determine the maximum allowable inside diameter of the brake drum or the 


     maximum “turn to” dimension.


Maximum allowable inside diameter = ___________  (allow 0.030” for wear)


Maximum “turn to” diameter = ___________

_____  4.  Measure the drum using a drum micrometer.



Left = __________     Right = ___________



OK to machine ______   NOT OK to machine ______
_____  5.  Based on the inspection, what is the necessary action? _________________________

     
_________________________________________________________________

