PAGE  
71
[image: image2.jpg]







Hydro-Boost Test

Meets NATEF Task:  (A5-E-4) Inspect and test hydro-boost system for leaks and proper 

operation. (P-3)

[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
_____  1.  Check the service information for the specified Hydro-Boost testing procedure for the

     vehicle being serviced.



_________________________________________________________________
_____  2.  Start the testing of a Hydro-boost power brake assist system by carefully inspecting 

     the following components:


Power steering fluid level


OK _____   NOT OK _____


Power steering pressure hoses for leaks
OK _____   NOT OK _____


Power steering pump drive belt

OK _____   NOT OK _____


Master cylinder brake fluid level

OK _____   NOT OK _____


Visually inspect the Hydro-boost assembly 


for evidence of power steering fluid leaks
OK _____   NOT OK _____

_____  3.  Check the operation of the base hydraulic brakes by depressing the brake pedal several 

     times with the engine "off" until the brake pedal feels firm.  Continue to apply force to 

     the brake pedal.  The brake pedal should not drop.


OK _____   NOT OK _____(master cylinder or hydraulic system fault is

     indicated)

_____  4.  With your foot still applying force to the brake pedal, start the engine.  If the 

     Hydro-boost system is functioning correctly, the brake pedal should drop.


OK _____   NOT OK _____

[image: image2.jpg]
_____  5.  To check the power steering pump for proper operation, 

     connect a power steering pressure gauge or pressure and 

     volume gauge between the pump and the Hydro-boost 

     unit.  Start the engine and observe the pressure and 

     volume gauges.


Pressure at idle = _________ 

(should be less than 150 psi) 

OK _____   NOT OK _____


Volume at idle = __________ 

(should be at least 2 gallons per minute)



OK _____   NOT OK _____

