PAGE  
93
[image: image2.jpg]







Tire Repair

Meets NATEF Task:  (A4-F-9) Repair tire using internal patch.  (P-1)

[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
_____  1.  Locate the source of the leak by submerging the tire under water or by spraying the 

     tire with soapy water.  Describe the location of the leak.


__________________________________________________

_____  2.  Remove the foreign object and use a reamer to clean the hole in the tire (tread area 

[image: image2.jpg]     only).

_____  3.  Dismount the tire and buff the inside of the tire around
     the hole.
_____  4.  Apply rubber cement to the buffed area.

_____  5.  Insert the repair plug from the inside of the tire.

_____  6.  Pull the plug through the puncture from the outside of the tire.

_____  7.  Use a stitching tool to make sure the inside of the patch is well adhered to the inside of 

     the tire.

_____  8.  Remove the tire and inflate to the air pressure specified by the vehicle manufacturer.

_____  9.  Check the repair for air leaks using soapy water.



OK ____    NOT OK ____

