PAGE  
52
[image: image3.jpg]NOT USED
IGNITION (87 A)

TERMINAL

BATTERY
TERMINAL ///////
(30) Z

\
GROUND

(85) SOLENOID
TERMINAL
(87)






Starter Relays and Solenoids
Meets NATEF Task:  (A6-C-3)  Inspect and test starter relays and solenoids and starter control circuit; determine necessary action.  (P-2)
[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
[image: image2.jpg]



_____  1.  Check service information for the specified procedure to follow when inspecting 

     starter control circuit wiring.

__________________________________________________________________



__________________________________________________________________
_____  2.  Set a digital multimeter (DMM) to read ohms (low scale) and check the hold-in coil 

                 and the pull-in coil.



Pull-in coil.  Measure between terminals “S” and “M”:



resistance = ______ (should be 0.2 to 0.4 ohm)  OK _____
NOT OK _____



Hold-in coil.  Measure between terminals “S” and the solenoid housing:

resistance = _______ (should be 0.4 to 0.6 ohm) OK _____
NOT OK _____

_____  3.  Test the pull-in winding by applying 12 volts to terminal “S” and ground to terminal

                 “M.”  Check that the plunger will be drawn into the solenoid.

[image: image3.jpg]
OK _____
NOT OK _____
_____  4.  Check the hold-in winding by connecting 12 volts

     to terminal “S” and the other wire to ground.  The 

     plunger should be drawn into the solenoid housing.




OK _____
NOT OK _____

_____  5.  Measure coil resistance of the relay (terminals 86 and
     85).



Resistance = ______ ohms 

   
(should be 60 to 100 ohms)
OK _____
NOT OK _____

_____ 6.  What is the necessary action?  


   
_____________________________________



_____________________________________

