PAGE  
33
[image: image2.jpg]




Engine Cooling System Inspection 
Meets NATEF Task:  (A1-D-3)  Perform cooling system pressure tests; check coolant condition; inspect and test radiator, pressure cap, coolant recovery tank, and hoses. (P-1)
[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
[image: image2.jpg]
_____  1.  Inspect the radiator for exterior leaks or clogged 


     areas due to bugs, dirt, or debris, and clean as 


     required.



OK _____
NOT OK _____

_____  2.  Pressure test the cooling system.  The entire 


     system should hold about 15 psi for 5 


     minutes, unless there is a leak.



OK _____
NOT OK _____   



Location of leak ____________________

_____  3.  Pressure test the radiator cap.  The cap holds _________ psi.



OK _____
NOT OK _____

_____  4.  Check the freezing point of the coolant using an hydrometer = __________


     [should be -34( F (-37( C) or lower].



OK _____
NOT OK _____

_____  5.  Check the boiling point of the coolant using an hydrometer = __________.



OK _____
NOT OK _____

