PAGE  
39
[image: image2.jpg]




Engine Fan Inspection and Testing
Meets NATEF Task: (A1-D-10)  Inspect and text fans(s) (electrical or mechanical), fan clutch, fan shroud, and air dams.  (P-1)
[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
[image: image2.jpg]
_____  1.  Identify the type of cooling fans.



____  Engine driven



____  Electric  (If electric, one or two fans? ____)



____  Hydraulically operated

_____  2.  If mechanical, check the fan clutch for fluid (silicone 

                 fluid) leakage or other damage.



OK _____    NOT OK _____  Describe the fault: _________________________

_____  3.  Check the mechanical fan clutch for proper operation by placing cardboard over the 

     front of the radiator and start the engine.  Allow the engine to reach normal operating 

     temperature.  As the temperature of the coolant increases above the normal 

     temperature range, the fan noise should increase indicating that the fan clutch has 

     engaged.


OK _____    NOT OK _____

_____  4.  Inspect the fan shroud for damage or if the shroud is missing.



OK _____    NOT OK _____

_____  5.  Inspect that the air dam underneath the front of the vehicle is in place and not missing 

     or damaged.


OK _____    NOT OK _____

_____  6.  From the inspection above, what is the necessary action?  ______________________


     _____________________________________________________________________
