PAGE  
29
[image: image2.jpg]TO STARTER TO STARTE
MOTOR MO'SFER R
- —

STARTING
VEHICLE

STALLED
VEHICLE

—
TO ENGINE
GROUND

s
TO ENGINE
GROUND

ENGINE BLOCK OR
METAL BRACKET
ON ENGINE BLOCK





Jump Starting
Meets NATEF Task:  (A6-B-6)  Start a vehicle using jumper cables and a battery or auxiliary power supply.  (P-1)
[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1          
_____  1.  Move the good vehicle to within reach of the jumper cables of the disabled vehicle (or 

                 use a portable jumper box).

_____  2.  Check that the ignition is in the “off” position on both vehicles.

_____  3.  Connect the jumper cables in the following order:



a.  Red cable ends to the positive (+) terminals of both batteries



b.  One black cable end to the negative (-) terminal of the good vehicle



c.  The other black cable end to a good, unpainted engine ground at least one 

                             foot from the battery

_____  4.  Start the good vehicle.

[image: image2.jpg]
_____  5.  Allow the good vehicle to run for 

     several minutes to charge the battery 

     of the disabled vehicle.

_____  6.  Start the disabled vehicle.

_____  7.  After the engine is running smoothly, 

     disconnect the black cable from the 

     engine ground of the disabled vehicle 

     and then disconnect the negative 

     terminal of the battery of the good 

     vehicle.

_____  8.  Disconnect the red battery cable ends from the positive terminals of the batteries.

