PAGE  
101
[image: image2.jpg]Oil
pressure gauge

Qil pressure
sending unit hole





Oil Pressure Measurement 
Meets NATEF Task:  (A1-D-1)  Perform oil pressure test; determine necessary action.  (P-1)
[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
_____  1.  Locate the oil pressure-sending (sender) unit.

_____  2.  Remove the sending unit using the proper size sending unit socket or wrench.

_____  3.  Thread a mechanical oil pressure gauge into the thread portion of the engine block 

                 where the sending unit was located.

_____  4.  Route the oil pressure gauge hose away from the moving components of the engine.

[image: image2.jpg]
_____  5.  Start the engine and check for leaks.

_____  6.  Record the oil pressure:



oil pressure @ idle _____________



oil pressure @ 1,000 RPM ________



oil pressure @ 2,000 RPM ________



oil pressure @ 3,000 RPM ________

NOTE:  Most engines require about 

10 psi per 1,000 RPM.


_____  7.  Results:  (check one)



great_____ (over 10 psi per 1,000 RPM)



good _____ (at 10 psi per 1,000 RPM)



bad  _____ (less than 10 psi per 1,000 RPM)

_____  8.  Based on the test results, what is the necessary action?  ________________________



__________________________________________________________________
