PAGE  
115
[image: image1.jpg]We Suppont

EEEEEEEEE





Cylinder Head Disassembly
Meets NATEF Task:  (A1-B-1)  Remove cylinder head(s); visually inspect for cracks; check gasket surface areas for leakage; check passage condition. (P-2)
[image: image1.jpg]Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
Engine ___________________ Displacement _____________________

_____  1.  Remove the camshaft(s) if overhead camshaft design.

_____  2.  Remove the rocker arm/cam followers and place them in order so that the parts can be 

     assembled in the same location.

NOTE:   All parts wear during operation and each part tends to wear slightly differently, so this is an important step that assures proper reassembly.

_____  3.  Remove the hydraulic lifter/lash adjuster, if equipped, and place them so that they can 

     be reinstalled in the same location.

_____  4.  Use a dead blow (plastic shot-filled) hammer and tap the valve spring retainer to 

   
     "break the taper" of the valve keepers (locks).

_____  5.  Use a valve spring compressor to compress the valves and remove the keepers.  

     Remove the valve from the cylinder head and place the valve, spring, retainer, and 

     keepers together for inspection and reassembly.

_____  6.  Are valve rotators used on this engine?



_____ Yes (If yes, where are they located?)  ______________________________

____________________________________________________________

 

_____ No

_____  7.  Remove the valve stem seals and spring seat (if aluminum head).

_____  8.  Type of valve stem seals?  

_____ positive (over guide)



_____ umbrella (on valve stem)



_____ O-ring (in groove of valve)

_____ part of the valve spring seat
