PAGE  
144
[image: image2.jpg]TOP RAIL

EXPANDER

BOTTOM RAIL

END GAP





Piston Ring Specification and Measurement 
Meets NATEF Task:  (A1-C-12)  Inspect, measure and install piston rings.  (P-2)
[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
_____  1.  Install each ring into the designated cylinder by inverting the piston and pushing each 

                 ring into the cylinder.  Measure the piston ring gap (should be about 0.004 in. for each 


     inch of bore).





               Specification          Actual           OK     NOT OK



Top ring                         _____________  ____________  _____   _____



2nd compression ring    _____________  ____________  _____   _____



Oil control ring              _____________  ____________  _____   _____

[image: image2.jpg]


_____  2.  File the butt ends of the rings as 


     necessary to achieve proper ring gap.

_____  3.  Install the piston rings on the pistons.

_____  4.  Measure the piston ring side clearance:  


     (usually about 0.001-0.003 in.)






    Specification          Actual           OK     NOT OK



Top ring                         _____________  ____________  _____   _____



2nd compression ring    _____________  ____________  _____   _____



Oil control ring              _____________  ____________  _____   _____

_____  5.  Push on the face of all piston rings checking to make sure that they go fully into the 

                 ring groove, so that the ring face is flush or slightly recessed with the surface of the 


     piston ring lands.



OK _____     NOT OK _____
