PAGE  
146
[image: image2.jpg]SURFACE
PLATE

BEND
INDICATOR

CONNECTING
ROD
CONTACT
BARS





Connecting Rod Reconditioning 
Meets NATEF Task:  (Not specified by NATEF)
[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
[image: image2.jpg]_____  1.  Check the connecting rod for twist.



#1 __________

#5 __________



#2 __________

#6 __________



#3 __________

#7 __________



#4 __________

#8 __________



OK _____      NOT OK _____

_____  2.  If not OK, were they able to be straightened?  Yes _____    No _____

_____  3.  Install the rod caps and torque to specification.

NOTE:  If new replacement connecting rod bolts are going to be used, press in new bolts before reconditioning the connecting rod.

_____  4.  Measure the roundness of the big end of the rod: ______ (should be less than 0.001")

_____  5.  Adjust the rod gauge to zero for the proper rod size.

_____  6.  Remove the connecting rod caps and grind 0.001" - 0.002" from the mating surface of 

     the caps.

_____  7.  Reassemble the caps onto the rod and torque the fasteners to factory specification.

_____  8.  Hone the inside diameter of the connecting rod back to its original diameter and 

     within specification for out-of-round and taper.

HINT:  Place two rods together to help maintain a true inside diameter with less chance of creating a tapered hole.

_____  9.  How close to actual specification were all connecting rods? ___________________



OK _____     NOT OK _____
