PAGE  
169
[image: image1.jpg]We Suppont

EEEEEEEEE





Cylinder Head Replacement
Meets NATEF Task:  (A1-B-2)  Install cylinder heads and gaskets; tighten according to the manufacturer’s specifications and procedures.  (P-1)
[image: image1.jpg]Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
The cylinder head gasket will have to be replaced if there is one or more of the following problems:

· An internal coolant leak into the combustion chamber

· An external oil or coolant leak at the head gasket

_____  1.  Allow the engine to cool, and drain the coolant into a suitable container to be disposed 

     of properly or recycled.

_____  2.  Remove the air cleaner assembly and other components as necessary to gain access to 

     the cylinder head and retaining bolts.  Items that required removal: 

_____________________  ___________________  ____________________


_____________________  ___________________  ____________________

_____  3.  Remove the intake manifold and cylinder head(s) following the tightening torque 

     sequence in reverse.

_____  4.  Clean all gasket surfaces.

CAUTION:  Do not use fiber abrasive pads to clean the gasket surfaces.  Particles of the fiber disc can get into the engine and cause serious engine wear and damage.  Do not use steel tools to scrape gaskets from an aluminum surface.

_____  5.  Install the replacement cylinder head and intake manifold gaskets following the 

                 vehicle manufacturer’s recommended procedure.  

_____  6.  Torque the retaining bolts to factory specifications.  ___________________________



Intake manifold bolt torque specification = ____________________



Cylinder head bolt torque specification = ___________________

_____  7.  Reassemble the top of the engine.

_____  8.  Refill the cooling system with new coolant and check for leaks and proper engine 

     operation.

CAUTION:  Be sure to open cooling system bleeder valves(s), if equipped, to avoid trapping air.
