PAGE  
52
[image: image2.jpg]DISTRIBUTOR

OHMMETER
s o CAP
'v"j: OMMETER
Il LEADS
® &






Spark Plug Wires
Meets NATEF Task:  (A8-C-2)  Inspect and test ignition primary and secondary circuit wiring and solid state components; test ignition coil(s); perform necessary action.  (P-1)
[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
_____  1.  Check service information to determine the specified spark plug wire resistance 

     specification.


_________________________
_____  2.  Set the digital multimeter to read ohms (().  (200 k if normal range meter)

_____  3.  Carefully disconnect both ends of a spark plug wire (there is no need to remove the 


     wire if both ends can be reached).

_____  4.  List the length in feet and the resistance values in ohms for each spark plug wire 

                 according to the cylinder number:

[image: image2.jpg]
          
    Length (feet)       Ohms



1. ___________    __________



2. ___________    __________

3. ___________    __________



4. ___________    __________



5. ___________    __________



6. ___________    __________



7. ___________    __________



8. ___________    __________

  
Coil wire (if equipped): ____________________    ___________________

_____  5.  Results - Usually 10,000 ohms (10 k() or less per foot of length.




OK_____   NOT OK_____
