PAGE  
89
[image: image2.jpg]




Oil Leak Detection
Meets NATEF Task:  (A1-A-5)  Inspect engine for fuel, oil, coolant and other leaks; determine necessary action (P-1)

[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
Engine oil is usually amber in color when new, but quickly becomes darker and often black when used in an engine.  Before trying to repair an engine oil leak, make sure that the leak is actually engine oil and not some other fluid such as the following:

· red – automatic transmission fluid (also used in some power steering units)

· [image: image2.jpg]green – antifreeze coolant

· orange – antifreeze coolant

· blue – antifreeze coolant or windshield washer fluid

· yellow – windshield washer fluid

· clear – condensation from the air-conditioning system (normal)

_____  1.  Carefully inspect the areas where oil is likely to leak, including:



valve covers
                
OK _____     NOT OK _____


intake manifold area        
OK _____     NOT OK _____


oil pressure-sending unit 
OK _____     NOT OK _____

_____  2.  Safely hoist the vehicle and carefully inspect the underneath of the engine.

_____  3.  Where is the highest, most forward area of the leak? (describe)  ________________


     ___________________________________________________________________

_____  4.  If the exact location cannot be located, lower the vehicle and add fluorescent dye to 


     the engine oil.  Drive the vehicle for 10 to 15 minutes and hoist the vehicle.

_____  5.  Using black light, locate the area of the leak by looking for the yellow/green areas

     highlighted by the dye.  Describe the leak location:


     ____________________________________________________________
_____  6.  Based on the inspection for oil leaks, what is the necessary action? _______________


     _____________________________________________________________________
