Automotive Engines
Chapter 5 Power Tools & Shop Equipment
Opening Your Class

	KEY ELEMENT
	EXAMPLES

	Introduce Content
	This engine systems course or class provides complete coverage of the components, operation, design, and troubleshooting. It correlates material to task lists specified by ASE and NATEF and emphasizes a problem-solving approach. Chapter features include Tech Tips, Frequently Asked Questions, Real World Fixes, Videos, Animations, and NATEF Task Sheet references. service. It correlates material to task lists specified by ASE and NATEF.    

	Motivate Learners
	Explain how the knowledge of how something works translates into the ability to use that knowledge to figure why the engine does not work correctly and how this saves diagnosis time, which translates into more money.

	State the learning objectives for the chapter or course you are about to cover and explain this is what they should be able to do as a result of attending this session or class.
	Explain the chapter learning objectives to the students.  

1.  Identify commonly used power tools.

2.  Identify commonly used shop equipment.

3.  Discuss the proper use of power tools and shop equipment.

4.  Describe the safety procedures that should be followed when working with power tools and shop equipment.

	Establish the Mood or Climate
	Provide a WELCOME, Avoid put downs and bad jokes. 

	Complete Essentials
	Restrooms, breaks, registration, tests, etc.

	Clarify and Establish Knowledge Base
	Do a round robin of the class by going around the room and having each student give their backgrounds, years of experience, family, hobbies, career goals, or anything they want to share.


	ICONS
	Ch5 Power Tools & Shop Equipment

	[image: image1.jpg]



	1. SLIDE 1 Ch 5 Power Tools & Shop Equipment

2. SLIDES 2-3 EXPLAIN objectives & KEY TERMS

4. SLIDE 4 EXPLAIN Air Compressor

	[image: image2.jpg]


[image: image3.jpg]



	Check for ADDITIONAL VIDEOS & ANIMATIONS @ http://www.jameshalderman.com/ 

WEB SITE REGULARLY UPDATED

	[image: image4.jpg]



	5. SLIDE 5 EXPLAIN FIGURE 5-1 typical shop compressor. It is usually placed out of the way, yet accessible to provide for maintenance to the unit.

6. SLIDE 6 EXPLAIN FIGURE 5-2    Always use an air nozzle that is OSHA approved. The openings in the side are used to allow air to escape if the nozzle tip were to become clogged. 

	[image: image5.jpg]


 
	Show VIDEO on Shop Air 

1 minute www.myautomotivelab.com 

http://media.pearsoncmg.com/ph/chet/chet_mylabs/akamai/template/video640x480.php?title=Compressed%20Air&clip=pandc/chet/2012/automotive/Auto_Shop_Safety/Clip25CompAir1.mov&caption=chet/chet_mylabs/akamai/2012/automotive/Auto_Shop_Safety/xml/Clip25CompAir1.xml

	[image: image6.jpg]



	DEMONSTRATION: Show shop’s air compressor & discuss how it works. What types of power tools can be used with the air compressor? What are some other applications? 

	[image: image7.jpg]



[image: image8]
	SAFETY Review safety procedures for using an air compressor & power tools associated with it. Air tools are powerful & can cause injury if not used properly

	[image: image9.jpg]



[image: image10]
	SAFETY Never point an air blow gun at yourself or anyone else.

	[image: image11.jpg]



	NEVER use compressed air to spin a bearing or a gear to make a whistling sound

	[image: image12.jpg]



	7.  SLIDE 7 EXPLAIN Air and Electrically Operated Tools

8.  SLIDE 8 EXPLAIN FIGURE 5-3 typical 1/2 in. drive impact wrench

9.  SLIDE 9 EXPLAIN FIGURE 5-4    This impact wrench features a variable torque setting using a rotary knob. The direction of rotation can be changed by pressing the button at the bottom.

10.  SLIDE 10 EXPLAIN FIGURE 5-5 typical battery-powered 3/8 in. drive impact wrench.

11.   SLIDE 11 EXPLAIN FIGURE 5-6 black impact socket. Always use impact-type sockets whenever using an impact wrench to avoid the possibility of shattering the socket, which can cause personal injury.

	[image: image13.jpg]



	12.  SLIDE 12 EXPLAIN FIGURE 5-7 air ratchet is a very useful tool that allows fast removal and installation of fasteners, especially in areas that are difficult to reach or do not have room enough to move a hand ratchet wrench.

	[image: image14.jpg]



	DEMONSTRATION: Show students how to use an impact wrench, and discuss its applications.


	[image: image15.jpg]



	HANDS-ON TASK: Have students use an

impact wrench and impact sockets to

remove and replace a car’s tires.

	[image: image16.jpg]



[image: image17]
	SAFETY Remind students they should ALWAYS wear eye protection when using power tools and other shop equipment.

	[image: image18.jpg]



	13.  SLIDE 13 EXPLAIN FIGURE 5-8    This typical die grinder surface preparation kit includes the air-operated die grinder, as well as a variety of sanding discs for smoothing surfaces or removing rust.


	[image: image19.png]



	14.  SLIDE 14 EXPLAIN REAL WORLD FIX

	[image: image20.jpg]



	15.  SLIDE 15 EXPLAIN FIGURE 5-9    A  fluorescent trouble light operates cooler and is safer to use in the shop because it is protected against accidental breakage where gasoline or other flammable liquids would happen to come in contact with the light. 

	[image: image21.jpg]


[image: image22.jpg]



	DISCUSSION: Have students talk about types of trouble lights found in shop. Why are fluorescent lights safer than incandescent lights? What are applications of various types of trouble lights?

	[image: image23.jpg]



	16.  SLIDE 16 EXPLAIN WARNING

	[image: image24.jpg]



	17.  SLIDE 17 EXPLAIN Bench/Pedestal Grinder

18.  SLIDE 18 EXPLAIN FIGURE 5-10 typical pedestal grinder with a wire wheel on left side and stone wheel on the right side. Even though this machine is equipped with guards, safety glasses or a face shield should always be worn when using a grinder or wire wheel

	[image: image25.jpg]



	HANDS-ON TASK: Have students use bench- or pedestal mounted grinder to clean bolt threads or sharpen a chisel. Warn students that they must wear a face shield when working with a grinder.

	[image: image26.jpg]



	19.  SLIDE 19 EXPLAIN Bench Vise & EXPLAIN FIGURE 5-11 typical vise mounted to a workbench 

	[image: image27.jpg]


[image: image28.jpg]



	DISCUSSION: Ask students to discuss how a bench vise works. What are its various uses in an automotive shop? FIGURE 5-11

	[image: image29.jpg]



	20.  SLIDE 20 EXPLAIN Hydraulic Presses & EXPLAIN FIGURE 5-12 hydraulic press is usually used to press bearings on and off on rear axles and transmissions 

	[image: image30.jpg]


 
	SHOW VIDEO ON Hydraulic press  1 MINUTE
http://media.pearsoncmg.com/ph/chet/chet_mylabs/akamai/template/video640x480.php?title=Hydraulic%20Press&clip=pandc/chet/2012/automotive/Auto_Shop_Safety/Clip40HydrPress1.mov&caption=chet/chet_mylabs/akamai/2012/automotive/Auto_Shop_Safety/xml/Clip40HydrPress1.xml   

	[image: image31.jpg]



	DEMONSTRATION: SHOW how to use a hydraulic press, have them press bearings on & off by using a bearing splitter FIGURE 5-12

	[image: image32.jpg]



	HANDS-ON TASK: After showing students

how to use a hydraulic press, have them press bearings on & off by using a bearing splitter FIGURE 5-12

	[image: image33.jpg]



[image: image34]
	SAFETY Be sure the rest and the safety shield are properly adjusted before using the bearing splitter

	[image: image35.jpg]



	Cover bearings with a towel or shop rag when pressing them to prevent pieces from

scattering if the bearing binds or explodes

	[image: image36.jpg]



	21.  SLIDE 21 EXPLAIN Portable Crane and Chain Hoist & EXPLAIN FIGURE 5-13 typical portable crane used to lift and move heavy assemblies, such as engines and transmissions 

	[image: image37.png]



	22.  SLIDE 22 EXPLAIN TECH TIP

	[image: image38.jpg]



	23.  SLIDE 23 EXPLAIN Engine Stands & EXPLAIN FIGURE 5-14 Two engines on engine stands. The plastic bags over engines help keep dirt from getting onto these engines and engine parts.

24.  SLIDE 24 EXPLAIN FIGURE 5-15 engine stand that grasps engine from the sides rather than end

	[image: image39.jpg]



	25.  SLIDE 25 EXPLAIN Care and Maintenance of Shop Equipment 

	[image: image40.jpg]



	A clean shop is a safer shop and may also attract more customers.

	[image: image41.jpg]



	Talk through SUMMARY and questions

	[image: image42.jpg]



	HOMEWORK: complete Ch5 crossword puzzle: http://www.jameshalderman.com/links/book_engine_theory_serv_7/cw/crossword_ch_5.pdf    


[image: image43.png]


