PAGE  
43
[image: image3.jpg]







Wheel Bearing Service

Meets NATEF Task:  (A4-D-2) Remove, inspect, and service or replace front and rear wheel bearings.  (P-1)

[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
[image: image2.jpg]



_____  1.  Remove the wheel cover and the hub 


     dust cap (grease cap).

[image: image3.jpg]
_____  2.  Remove and discard the cotter key.

_____  3.  Remove the spindle nut, washer and outer 


     bearing.

_____  4.  Remove inner and outer bearing and grease 

 
     seal.

_____  5.  Thoroughly clean the bearing in solvent and 


     denatured alcohol or brake cleaner and blow 


     it dry with compressed air.

_____  6.  Closely inspect the bearing for wear or


     damage.

_____  7.  Show the instructor the cleaned bearing.      
Instructor's OK _______________

_____  8.  Repack the bearing with the correct type of wheel bearing grease.

_____  9.  Install a new grease seal using a seal installing tool.

_____ 10. Correctly adjust the bearing preload.  State the specified procedure.  ______________



_________________________________________________________________

_____ 11.  Install the hub dust cap (grease cap) and wheel cover.

_____ 12.  If a sealed wheel bearing is being replaced, what is the 

      specified procedure and torque values?


________________________________________________



________________________________________________

