PAGE  
46
[image: image2.jpg]







Steering and Suspension Concerns

Meets NATEF Task:  (A4-E-1) Diagnose vehicle wander, drift, pull, hard steering, bump steer, memory steer, torque steer; determine necessary action.  (P-1)
[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
[image: image2.jpg]
_____  1.  Check the service information to determine the alignment specifications.



Camber = __________    Caster = __________    Toe = ___________
_____  2.  Hoist the vehicle on the alignment rack and install the wheel sensors.

_____  3.  Compensate the wheel sensors.

_____  4.  Lower the vehicle and jounce (bounce) to center the suspension.

_____  5.  Read the rear camber and toe.




   
 LR

  RR


Camber 
_____

 _____



Toe       
_____

 _____



Total rear toe =    _______

_____  6.  Read the front camber and toe.


   


  LF

  RF


Camber 
_____

 _____



Toe       
_____

 _____



Total front toe =    _______

_____  7.  Perform a caster sweep to determine the front caster and SAI.


  


  LF

  RF


Caster  
_____

 _____



SAI       
_____

 _____



Based on the alignment angles, what action is needed?  _____________________



__________________________________________________________________

