PAGE  
37
[image: image2.jpg]ADDOIL —f o FuLL CAUTION 00}

ADD | — SAFE f—  WARRANTY REQ. |

Aco oL e FucL Kt

& E 0






Engine Oil Dipstick Test

Meets NATEF Task:  (A1-A-7)  Diagnose oil consumption concerns; determine necessary action.     (P-2)
[image: image1.jpg]We Suppont

EEEEEEEEE




[image: image2.jpg]Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
_____ 1.  Check the engine oil level and condition.



Level:  ____low     ____okay     ____overfilled



Color:  ____black   ____dark brown     ____amber


_____  2.  Ignite the oil on the dipstick:



_____ does not burn



_____ does burn

NOTE:  If the oil on the dipstick burns, there is gasoline in the oil.  This will 

cause the engine to run rich by drawing the fuel through the PCV system.  

Gasoline in the oil can also damage the engine due to the diluted oil not properly 

lubricating the engine.

_____ 3.  Drip oil onto the hot exhaust manifold:



_____ does not bubble



_____ does bubble

NOTE:  If the oil bubbles on the exhaust manifold, coolant (water) is in the oil.

_____ 4.  Rub the oil from the dipstick between your fingers:



_____ does not feel gritty



_____ does feel gritty

_____  5.  Based on the test results, what is the necessary action? _________________________



__________________________________________________________________
