PAGE  
39
[image: image2.jpg]




Accessory Drive Belt Inspection

Meets NATEF Task:  (A1-D-4)  Inspect and adjust drive belts; check pulley alignment.  (P-1)
[image: image1.jpg]We Suppont

EEEEEEEEE




[image: image2.jpg]Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1        
Proper operation of the alternator and the charging system as well as the water (coolant) pump and other cooling systems depend on the accessory drive belt(s).

Serpentine (poly-V) belt:
_____  1.  Check for cracks - replace if more than 3 cracks in any one rib in 3 inches.

_____  2.  Check for proper tension - check the tensioner notch location for proper tension 

     position.


OK ____   NOT OK ____

V-belts:
_____  1.  Check belt tension:

A.  Usual specifications is 70 to 100 lb. of tension using a belt tension gauge.

B.  Maximum of ½” deflection.



OK ____   NOT OK ____
_____  2.  Check the belt for flaying, cracks and glazing.



OK ____   NOT OK ____
_____  3.  Check pulley adjustment.         


OK ____   NOT OK ____
