# Automotive Electrical & Engine Performance 7/E

# Chapter 30 MAP/BARO SENSORS

## Opening Your Class

| KEY ELEMENT | EXAMPLES |
| --- | --- |
| Introduce Content | This course or class covers Automotive Electrical & Engine Performance. It correlates material to task lists specified by ASE and NATEF. |
| Motivate Learners | Explain how the knowledge of how something works translates into the ability to use that knowledge to figure why the engine does not work correctly and how this saves diagnosis time, which translates into more money. |
| State the learning objectives for the chapter or course you are about to cover and explain this is what they should be able to do as a result of attending this session or class. | Explain the chapter learning objectives to the students. - Discuss the variations in pressure that can occur within an engine. - Discuss how MAP sensors work. - List the methods that can be used to test MAP sensors. - Describe the symptoms of a failed MAP sensor. - List how the operation of the MAP sensor affects vehicle operation. - Discuss MAP sensor rationality tests. - Describe how the BARO sensor is used to test altitude. This chapter will help you prepare for Engine Repair (A8) ASE certification test content area “E” (Computerized Engine Controls Diagnosis and Repair). |
| Establish the Mood or Climate | Provide a WELCOME, Avoid put downs and bad jokes. |
| Complete Essentials | Restrooms, breaks, registration, tests, etc. |
| Clarify and Establish Knowledge Base | Do a round robin of the class by going around the room and having each student give their backgrounds, years of experience, family, hobbies, career goals, or anything they want to share. |

# NOTE: This lesson plan is based on Automotive Electrical & Engine Performance 7/E Chapter Images found on Jim’s web site @ www.jameshalderman.com

# LINK CHP 30: Chapter Images

| ICONS | Ch30 MAP/BARO SENSORS |
| --- | --- |
|  | 1. SLIDE 1 CH30 MAP/BARO SENSORS |
|  | Check for ADDITIONAL VIDEOS & ANIMATIONS @ http://www.jameshalderman.com/ WEB SITE REGULARLY UPDATED |
|  | Videos |
|  | At the beginning of this class, you can download the crossword puzzle & Word Search from the links below to familiarize your class with the terms in this chapter & then discuss them |
|  | Crossword Puzzle (Microsoft Word) (PDF) Word Search Puzzle (Microsoft Word) (PDF) |
|  | 2. SLIDE 2 EXPLAIN Figure 30-1 (a) As an engine is accelerated under a load, the engine vacuum drops. This drop in vacuum is actually an increase in absolute pressure in the intake manifold. A MAP sensor senses all pressures greater than that of a perfect vacuum. (b) The relationship between absolute pressure, vacuum, and gauge pressure. |
|  | DISCUSSION: Have the students discuss intake manifold pressure. How and why does throttle angle affect intake manifold vacuum? Discuss the difference between PSIG & PSIA. How is a perfect vacuum indicated in gauge pressure? How is atmospheric pressure, or barometric pressure, indicated in absolute pressure? FIGURE 30-1 |
|  | DISCUSSION: Have the students discuss the difference between MAP, BARO, & BMAP sensors. Is there any advantage to using separate MAP & BARO sensors? |
|  | 3. SLIDE 3 EXPLAIN Figure 30-2 A clear plastic MAP sensor used for training purposes showing the electronic circuit board and electrical connections |
|  | DEMONSTRATION: Show what a MAP sensor looks like and discuss where it can be found on most vehicles. FIGURE 30-2 |
|  | DISCUSSION: Have the students compare and contrast different types of pressure sensors (silicon diaphragm, capacitor capsule, & ceramic disc). Which is most commonly used design for a MAP sensor? |
|  | 4. SLIDE 4 EXPLAIN Figure 30-3 MAP sensors use three wires: 1. 5-volt reference from the PCM 2. Sensor signal (output signal) 3. Ground. A DMM set to test a MAP sensor. (1) Connect the red meter lead to the V meter terminal and the black meter lead to the COM meter terminal. (2) Select DC volts. (3) Connect the test leads to the sensor signal wire and the ground wire. (4) Select hertz (Hz) if testing a MAP sensor whose output is a varying frequency; otherwise keep it on DC volts. (5) Read the change of voltage (frequency) as the vacuum is applied to the sensor. Compare the vacuum reading and the frequency (or voltage) reading to the specifications. |
|  | DISCUSSION: Have the students discuss frequency. What is frequency? How is it measured? |
|  | HANDS-ON TASK: Have the students use a DMM to monitor MAP sensor frequency. FIGURE 30-3 |
|  | 5. SLIDE 5 EXPLAIN Figure 30-4 A waveform of a typical digital MAP sensor. |
|  | DEMONSTRATION: Show the students how to use a DSO to monitor MAP sensor frequency. Show them how frequency changes with changes in engine load. FIGURE 30-4 |
|  | 6. SLIDE 6 EXPLAIN Figure 30-5 Shown is the electronic circuit inside a ceramic disc MAP sensor used on many Chrysler engines. The black areas are carbon resistors that are applied to the ceramic, and lasers are used to cut lines into these resistors during testing to achieve the proper operating calibration |
|  | EXPLAIN TECH-TIP |
|  | DISCUSSION: Have students discuss EGR system operation. How could a leaking EGR pintle affect MAP sensor readings? |
|  | 7. SLIDE 7 EXPLAIN Figure 30-6 Altitude affects the MAP sensor voltage. |
|  | DISCUSSION: Have the students discuss how intake manifold vacuum leaks affect MAP sensor readings. How might this problem impact fuel economy and emissions? |
|  | DISCUSS REAL WORLD FIX |
|  | Older GM products that used MAP & BARO sensors used different color connectors to help technicians tell one from other. |
|  | DISCUSSION: Have the students discuss what a BARO sensor detects. How does a reduction in barometric pressure affect engine operation? |
|  | EXPLAIN TECH-TIP |
|  | HANDS-ON TASK: Have the students use a scan tool to monitor MAP sensor operation. |
|  | Map Sensor Ground Check Map Sensor Reference Voltage Check Map sensor Signal Voltage Check |
|  | 8. SLIDE 8 EXPLAIN Testing MAP Sensor Figure 30-7 hand-operated vacuum pump |
|  | DEMONSTRATION: Use a vacuum pump hooked up to MAP sensor & scan tool to show students how changes in engine load (manifold vacuum) affect pulse width (air-fuel mixture). FIGURE 30-7 |
|  | DISCUSSION: Have the students discuss how increases and decreases in fuel rail pressure affect injector pulse width. Why does this happen? |
|  | ON-VEHICLE NATEF TASK Inspect and test MAP Sensor using a GMM)/(DSO); perform necessary action |