PAGE  
113
[image: image2.jpg]




Service EGR System
Meets NATEF Task:  (A8-E-7)  Inspect, test, service and replace components of the EGR system, exhaust passages, vacuum/pressure controls, filters and hoses; perform necessary action. (P-1)
[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1           
The EGR passages and valve that control the flow of exhaust gases can become clogged with carbon.  The EGR valve and passages may need to be cleaned if one or more of the following conditions are present.

· A computer diagnosis trouble code (DTC) indicating the lack of EGR flow

· The failure of an exhaust emission test for excessive NOx

· Excessive engine spark knock (ping or detonation)

_____  1.  Check service information for the recommended procedures to follow when servicing 

     the EGR system.


_________________________________________________________________
_____  2.  What problem(s) exists? _________________________________________________

_____  3.  Remove the EGR valve and inspect for clogged passages.  Clean as needed.



Valve was clogged _____     Valve was OK _____

_____  4.  Start the engine.  Exhaust should be heard and felt coming from the open passage 

     where the EGR valve was located.

CAUTION:  Be sure to wear eye protection.  Particles of carbon can be forced out of the EGR passage with great force when the engine starts.



Exhaust flowed freely _____    Exhaust did not flow freely _____

[image: image2.jpg]_____  5.  To clean the passages of carbon, remove the plugs or 

                 EGR valve and insert a stiff wire into an electric drill 

     and use it to ream out the passages.

_____  6.  Reinstall the EGR valve with a new gasket and check 

     the engine for proper operation. 

_____  7.  What is the necessary action? ____________________



________________________________________

