PAGE  
45
[image: image1.jpg]We Suppont

EEEEEEEEE





Electronic Memory Saver Usage
Meets NATEF Task:  (A6-B-3)  Maintain or restore electronic memory functions.  (P-1)
[image: image1.jpg]Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1            
_____  1.  Check service information for the specified method and tools or equipment needed to 


     maintain or restore electronic memory function. 



__________________________________________________________________



__________________________________________________________________



__________________________________________________________________

_____  2.  Most domestic brand vehicles can use the cigarette (auxiliary outlet) to apply voltage


     when the battery of the vehicle is removed or disconnected furring service.  For this 


     procedure to work, the lighter or auxiliary outlet must be powered with the ignition 


     switch in the on position.



a.  Using a DMM set to read DC volts, check for battery voltage at the lighter 



     socket with the ignition off.    ____OK    ____ NOT OK


b.  Check for DC volts at the lighter socket with the ignition set to the on (run) 



     position.  ____ OK    ____ NOT OK
_____  3.  If the vehicle is equipped with a to-post-type battery design, a jump box can be 

  
     attached to the ends of the cables to keep the memory in the electronic components 


     while the battery is removed.  Is this a possible method? 



 ____ Yes    ____ No

_____  4.  Describe the method recommended to restore lost memory.  ____________________



_________________________________________________________________



_________________________________________________________________

