PAGE  
54
[image: image2.jpg]BATTERY

SOLENOID

7]
§f

STARTER





Starter Voltage Drop/Current Draw Tests
Meets NATEF Task:  (A6-C-1, A6-C-2, and A6-C-6)  Perform starter current draw tests; determine necessary action.  (P-1)
[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1           
_____  1.  Check service information for the specified starter current draw test procedure and 


     specifications.



__________________________________________________________________



__________________________________________________________________

NOTE:  Few vehicle manufacturers give starter current draw specifications with the starter installed on the vehicle.  Use the chart below as a guideline regarding the range of the maximum allowable starter draw.


4-cylinder engines = 150 to 185 amperes maximum (normally less than 100 A)


6-cylinder engines = 160 to 200 amperes maximum (normally less than 125 A)


8-cylinder engines = 185 to 250 amperes maximum (normally less than 150 A)

[image: image2.jpg]
_____  2.  Perform the starter current draw test following the 

                 manufacturer’s instructions.



Results:  ________ amperes


_____
3.  Connect the voltmeter, as shown in the 

                 illustration, and crank the engine.  Observe the 

                 voltmeter.

_____  4.  All test results should be less than 0.2 V (200 mV).

_____  5.  Based on the specifications and the test 


     results, what is the necessary action?



___________________________



___________________________



___________________________



___________________________

