PAGE  
85
[image: image2.jpg]




MAP Sensor Diagnosis
Meets NATEF Task:  (A8-B-7) Inspect and test sensors, actuators, and circuits using a graphing multimeter (GMM)/digital storage oscilloscope (DSO); perform necessary action.  (P-1)

[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1           
_____  1.  Check service information for the specified MAP sensor diagnosis procedure.



__________________________________________________________________ 

_____  2.  Perform a thorough visual inspection including:

a.   Check the condition of vacuum hose (if equipped).

b.   Check that the vacuum hose routing does not have any dips or sags in the 

      vacuum hose between the sensor and the intake manifold.


NOTE:  A dip or low portion in the vacuum hose can create a trap where liquid fuel 

            (condensed gasoline fumes) or water (condensed steam) can accumulate and block the 

            vacuum signal to the MAP sensor.

c.   Disconnect the vacuum hose (if equipped) from the MAP sensor.  If anything 

      such as a liquid or other substance comes out of the sensor or the hose, replace 

      the MAP sensor.  Reconnect the vacuum hose to the MAP.

_____  3.  Turn the ignition key on (engine off), read and record the MAP sensor voltage (or 


     frequency) = _____ volts (Hz) (use either a scan tool or digital meter connected to the 


     signal wire).  (Should be about 4.60 to 4.80 volts or 156-159 Hz.)



OK _____    NOT OK _____
_____  4.  Start the engine and operate until normal operating temperature is achieved.  Read and 


     record the MAP sensor voltage (or Hz) at idle speed = ________volts (Hz).  (Should 


     be between 0.9 and 1.6 volts (102-109 Hz) if the engine varies between 17 and 21 


     inches of Hg.)     OK _____  NOT OK _____
[image: image2.jpg] 

_____  5 .  Based on these tests, what is the necessary action? 



__________________________________________



__________________________________________



__________________________________________

