PAGE  
91
[image: image2.jpg]20KV 40 KV/V

15 KV 30 KV/V
FIRING
LINE
10KV H== 20 KV/V
SPARK LINE
(SPARK DURATION)
I INTERMEDIATE DWELL
SECTION SECTION
5 KV 10 KV/V
fr— 2
0KV L ! ! 1 I 0 KV/V

0 1mS 2ms 3ms 4 mS 5msS





Ignition Scope Analysis

Meets NATEF Task:  (A8-C-1) Inspect and test ignition primary and secondary circuit wiring and solid state components; test ignition coil(s); perform necessary action. (P-1)
[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1           
[image: image2.jpg]
_____  1.  Check service information regarding the 

     specified method for attaching and using 

     a secondary circuit oscilloscope.

_______________________________

_______________________________

_____  2.  Type of ignition:



_____  Distributor



_____  Waste spark



_____  Coil-on-plug

_____  3.  Connect the ignition scope to the system as per the scope manufacturer’s 

                 instructions.

_____  4.  Brand of scope used:  ________________________________________

_____  5.  Describe the hookup procedure. ___________________________________________

_____  6.  Start the engine and observe the secondary ignition waveform.



           

Firing Voltage (KV)


Spark Line Length (ms)




      (voltage should be 5-15 KV)
            (length should be 1-2 ms)

Cylinder #1          _______________________

_____________________


Cylinder #2          _______________________

_____________________


Cylinder #3          _______________________

_____________________


Cylinder #4          _______________________

_____________________


Cylinder #5          _______________________

_____________________


Cylinder #6          _______________________

_____________________


Cylinder #7          _______________________

_____________________


Cylinder #8          _______________________

_____________________

_____  7.  Based on the test results, what is the necessary action? _________________________

