PAGE  
93
[image: image2.jpg]




Spark Plugs Inspection

Meets NATEF Task:  (A8-C-4)  Inspect and test ignition primary and secondary circuit wiring and solid state components; test ignition coil(s); perform necessary action. (P-1)
[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1           
_____  1.  Check service information and determine the correct plug code and gap for your 


     vehicle using a spark plug application guide.  



Engine:  # Cylinders_____    VIN# ____________________

[image: image2.jpg]

Brand __________   Code # __________   Gap __________   

_____  2.  Remove and label all the spark plug wires.

_____  3.  Determine the condition and gap of all spark plugs:





Condition



Gap



1.  ______________________________________  _______



2.  ______________________________________  _______



3.  ______________________________________  _______



4.  ______________________________________  _______



5.  ______________________________________  _______



6.  ______________________________________  _______



7.  ______________________________________  _______



8.  ______________________________________  _______

_____  4.  Reinstall the spark plug (start by hand).

_____  5.  Use a torque wrench and torque the spark plugs to the proper torque.



Specified torque = ______________

_____  6.  Start the engine.  Check for possible rough running caused by crossed or loose spark 

                 plug wires.



OK _____
NOT OK _____
_____  7.  Based on the inspection of the spark plugs, what is the necessary action?  __________



__________________________________________________________________
