PAGE  
100
[image: image1.jpg]We Suppont

EEEEEEEEE





Port Fuel-Injection System Diagnosis

Meets NATEF Task:  (A8-D-1)  Diagnose hot or cold no-starting, hard starting, poor driveability, incorrect idle speed, poor idle, flooding, hesitation, surging; determine necessary action. (P-1)
[image: image1.jpg]Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1                   
_____  1.  Check service information for the recommended procedure to follow to diagnose the 


     fuel injection system.

_____  2.  Attach a fuel pressure gauge to the Schrader valve on the fuel rail, if available.

_____  3.  Turn the ignition key to "on" or start the engine to build up the fuel pump pressure.



_____psi (should reach specified fuel pressure, usually about 35-45 psi)

_____  4.  Turn the ignition off and wait 20 minutes and observe the fuel pressure retained in the 


     fuel rail = _______ psi.  If the drop is less than 20 psi in 20 minutes, everything is OK.

                 (The fuel pressure should not drop more than 20 psi in 20 minutes.) 



If the drop is greater than 20 psi in 20 minutes, there is a possible problem with:




a.  the check valve in the fuel pump.




b.  leaking injectors.




c.  a defective (leaking) fuel pressure regulator.



To determine which unit is defective, perform the following:




Step #1:
Re-energize the electric fuel pump.




Step #2:
Clamp the fuel supply line, wait 10 minutes.  If the pressure drop does not occur - replace the fuel pump.  If the pressure drop still occurs - continue with Step #3.




Step #3:
Repeat the pressure build up of the electric pump and clamp the fuel return line.  If the pressure drop time is now OK, replace the fuel pressure regulator.




Step #4:
If the pressure drop still occurs, the injectors are leaking.  Remove the injectors with the fuel rail and hold over paper.  Replace those injectors that drip a drop or more after 10 minutes with pressurized fuel.


CAUTION:  Do not clamp plastic fuel lines.  Connect shut-off valves to the fuel system 



          to shut off supply and return lines.

_____  5.  Based on the test results, what is the necessary action? _________________________                        


